Network Design &

VLSM Implementation

Step-by-Step VLSM Calculations Report

| Student Name | Muhammad Hassaan Bari |
| --- | --- |
| Assignment | 04 – Network Design & VLSM |
| Submission | April 15, 2026 |
| Base Network | 192.168.10.0 / 24 |
| Departments | 5 (Sales, HR, IT, Finance, Management) |
| Tool | Cisco Packet Tracer |

1.  Network Overview

This report documents the complete VLSM-based network design for a five-department organization built in Cisco Packet Tracer. The topology uses three routers (ISR4331 Router1, ISR4321 Router2, 1941 Router3), five department LANs each served by a dedicated switch, and three WAN point-to-point links interconnecting the routers. OSPF (Area 0) is used as the dynamic routing protocol across all links.

1.1  Topology Summary

| Department | Router | Switch | Hosts Required | Network IP |
| --- | --- | --- | --- | --- |
| Sales Dept | Router1 (ISR4331) | Sales Switch (2960-24) | 50 Hosts | 192.168.10.0 |
| HR Dept | Router2 (ISR4321) | HR Switch (2960-24) | 25 Hosts | 192.168.10.64 |
| IT Dept | Router3 (1941) | IT Switch (2960-24) | 12 Hosts | 192.168.10.96 |
| Finance Dept | Router3 (1941) | Finance Switch (2960-24) | 6 Hosts | 192.168.10.112 |
| Management Dept | Router3 (1941) | Management Switch (2960-24) | 2 Hosts | 192.168.10.120 |

2.  VLSM Calculations – Step by Step

Base Network: 192.168.10.0 / 24   Total Addresses: 256   Departments: 5

VLSM allocates subnets in descending order of host count — largest first — to prevent address space fragmentation. WAN point-to-point links are allocated last using /30 subnets (exactly 2 usable hosts).

2.1  Sorted Allocation Order

| # | Department | Hosts Required | Block Size (2ⁿ) | Prefix | Subnet Mask | Usable |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sales Department | 50 | 64 (2⁶) | /26 | 255.255.255.192 | 62 |
| 2 | HR Department | 25 | 32 (2⁵) | /27 | 255.255.255.224 | 30 |
| 3 | IT Department | 12 | 16 (2⁴) | /28 | 255.255.255.240 | 14 |
| 4 | Finance Department | 6 | 8 (2³) | /29 | 255.255.255.248 | 6 |
| 5 | Management Department | 2 | 4 (2²) | /30 | 255.255.255.252 | 2 |
| 6 | WAN Link 1 (R1↔R2) | 2 | 4 (2²) | /30 | 255.255.255.252 | 2 |
| 7 | WAN Link 2 (R2↔R3) | 2 | 4 (2²) | /30 | 255.255.255.252 | 2 |
| 8 | WAN Link 3 (R1↔R3) | 2 | 4 (2²) | /30 | 255.255.255.252 | 2 |

2.2  Subnet 1 — Sales Department  (/26)

| Parameter | Value |
| --- | --- |
| Hosts Required | 50 |
| Formula | 2ⁿ ≥ (50 + 2) = 52 → 2⁶ = 64 ≥ 52 ✓ (n = 6) |
| Prefix / Mask | /26 ¦ 255.255.255.192 |
| Network Address | 192.168.10.0 |
| First Usable Host | 192.168.10.1 |
| Last Usable Host | 192.168.10.62 |
| Broadcast Address | 192.168.10.63 |
| Usable Hosts | 62 (required: 50) ✓ |
| Next Available | 192.168.10.64 |

2.3  Subnet 2 — HR Department  (/27)

| Parameter | Value |
| --- | --- |
| Hosts Required | 25 |
| Formula | 2ⁿ ≥ (25 + 2) = 27 → 2⁵ = 32 ≥ 27 ✓ (n = 5) |
| Prefix / Mask | /27 ¦ 255.255.255.224 |
| Network Address | 192.168.10.64 |
| First Usable Host | 192.168.10.65 |
| Last Usable Host | 192.168.10.94 |
| Broadcast Address | 192.168.10.95 |
| Usable Hosts | 30 (required: 25) ✓ |
| Next Available | 192.168.10.96 |

2.4  Subnet 3 — IT Department  (/28)

| Parameter | Value |
| --- | --- |
| Hosts Required | 12 |
| Formula | 2ⁿ ≥ (12 + 2) = 14 → 2⁴ = 16 ≥ 14 ✓ (n = 4) |
| Prefix / Mask | /28 ¦ 255.255.255.240 |
| Network Address | 192.168.10.96 |
| First Usable Host | 192.168.10.97 |
| Last Usable Host | 192.168.10.110 |
| Broadcast Address | 192.168.10.111 |
| Usable Hosts | 14 (required: 12) ✓ |
| Next Available | 192.168.10.112 |

2.5  Subnet 4 — Finance Department  (/29)

| Parameter | Value |
| --- | --- |
| Hosts Required | 6 |
| Formula | 2ⁿ ≥ (6 + 2) = 8 → 2³ = 8 ≥ 8 ✓ (n = 3) |
| Prefix / Mask | /29 ¦ 255.255.255.248 |
| Network Address | 192.168.10.112 |
| First Usable Host | 192.168.10.113 |
| Last Usable Host | 192.168.10.118 |
| Broadcast Address | 192.168.10.119 |
| Usable Hosts | 6 (required: 6) ✓ |
| Next Available | 192.168.10.120 |

2.6  Subnet 5 — Management Department  (/30)

| Parameter | Value |
| --- | --- |
| Hosts Required | 2 |
| Formula | 2ⁿ ≥ (2 + 2) = 4 → 2² = 4 ≥ 4 ✓ (n = 2) |
| Prefix / Mask | /30 ¦ 255.255.255.252 |
| Network Address | 192.168.10.120 |
| First Usable Host | 192.168.10.121 |
| Last Usable Host | 192.168.10.122 |
| Broadcast Address | 192.168.10.123 |
| Usable Hosts | 2 (required: 2) ✓ |
| Next Available | 192.168.10.124 |

2.7  WAN Link Subnets  (/30)

All three router-to-router WAN links use /30 subnets. A /30 provides exactly 2 usable host addresses — one for each router interface on the link.

Block Size = 2² = 4  |  Usable Hosts = 2  |  Mask = 255.255.255.252

| WAN Link | Network | Router A IP | Router B IP | Broadcast | Prefix |
| --- | --- | --- | --- | --- | --- |
| WAN-1 R1 ↔ R2 | 192.168.10.124 | 192.168.10.125 | 192.168.10.126 | 192.168.10.127 | /30 |
| WAN-2 R2 ↔ R3 | 192.168.10.128 | 192.168.10.129 | 192.168.10.130 | 192.168.10.131 | /30 |
| WAN-3 R1 ↔ R3 | 192.168.10.132 | 192.168.10.133 | 192.168.10.134 | 192.168.10.135 | /30 |

3.  Complete VLSM Allocation Summary

The following table provides a complete summary of all 8 allocated subnets from the base network 192.168.10.0/24:

| Subnet | Network Addr | Subnet Mask | First Host | Last Host | Broadcast | Usable |
| --- | --- | --- | --- | --- | --- | --- |
| Sales /26 | 192.168.10.0 | 255.255.255.192 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 | 62 |
| HR /27 | 192.168.10.64 | 255.255.255.224 | 192.168.10.65 | 192.168.10.94 | 192.168.10.95 | 30 |
| IT /28 | 192.168.10.96 | 255.255.255.240 | 192.168.10.97 | 192.168.10.110 | 192.168.10.111 | 14 |
| Finance /29 | 192.168.10.112 | 255.255.255.248 | 192.168.10.113 | 192.168.10.118 | 192.168.10.119 | 6 |
| Mgmt /30 | 192.168.10.120 | 255.255.255.252 | 192.168.10.121 | 192.168.10.122 | 192.168.10.123 | 2 |
| WAN-1 /30 | 192.168.10.124 | 255.255.255.252 | 192.168.10.125 | 192.168.10.126 | 192.168.10.127 | 2 |
| WAN-2 /30 | 192.168.10.128 | 255.255.255.252 | 192.168.10.129 | 192.168.10.130 | 192.168.10.131 | 2 |
| WAN-3 /30 | 192.168.10.132 | 255.255.255.252 | 192.168.10.133 | 192.168.10.134 | 192.168.10.135 | 2 |

Total Addresses Used: 136 out of 256    Remaining Free Range: 192.168.10.136 – 192.168.10.255  (120 addresses available for future use)

4.  Device IP Address Assignment

The table below assigns specific IP addresses to every end device and router interface based on the VLSM subnets calculated above.

| Device | Department | IP Address | Subnet Mask | Prefix | Default GW |
| --- | --- | --- | --- | --- | --- |
| PC0 | Sales | 192.168.10.1 | 255.255.255.192 | /26 | 192.168.10.62 |
| PC1 | Sales | 192.168.10.2 | 255.255.255.192 | /26 | 192.168.10.62 |
| Router1 Fa0/0 | Sales GW | 192.168.10.62 | 255.255.255.192 | /26 | — |
| PC2 | HR | 192.168.10.65 | 255.255.255.224 | /27 | 192.168.10.94 |
| PC3 | HR | 192.168.10.66 | 255.255.255.224 | /27 | 192.168.10.94 |
| Router2 Fa0/0 | HR GW | 192.168.10.94 | 255.255.255.224 | /27 | — |
| PC4 | IT | 192.168.10.97 | 255.255.255.240 | /28 | 192.168.10.110 |
| PC5 | IT | 192.168.10.98 | 255.255.255.240 | /28 | 192.168.10.110 |
| Router3 Fa0/0 | IT GW | 192.168.10.110 | 255.255.255.240 | /28 | — |
| PC6 | Finance | 192.168.10.113 | 255.255.255.248 | /29 | 192.168.10.118 |
| PC7 | Finance | 192.168.10.114 | 255.255.255.248 | /29 | 192.168.10.118 |
| Router3 Fa0/1 | Finance GW | 192.168.10.118 | 255.255.255.248 | /29 | — |
| PC8 | Management | 192.168.10.121 | 255.255.255.252 | /30 | 192.168.10.122 |
| PC9 | Management | 192.168.10.122 | 255.255.255.252 | /30 | — |
| Router3 Fa1/0 | Mgmt GW | 192.168.10.122 | 255.255.255.252 | /30 | — |

5.  Routing Configuration

As per the assignment requirements, two static routes are manually configured and OSPF (Area 0) dynamic routing is used for the remaining networks to ensure full inter-department connectivity.

5.1  Static Routes

Two static routes are configured manually:

- Router1 → route to HR subnet (192.168.10.64/27) via Router2 WAN IP (192.168.10.126)

- Router2 → route to Sales subnet (192.168.10.0/26) via Router1 WAN IP (192.168.10.125)

! Static Route on Router1

Router1(config)# ip route 192.168.10.64 255.255.255.224 192.168.10.126

! Static Route on Router2

Router2(config)# ip route 192.168.10.0 255.255.255.192 192.168.10.125

5.2  OSPF Dynamic Routing (Area 0)

OSPF process 1 is configured on all three routers to advertise and learn all remaining subnets dynamically:

! Router1 OSPF

Router1(config)# router ospf 1

Router1(config-router)# network 192.168.10.0 0.0.0.63 area 0

Router1(config-router)# network 192.168.10.124 0.0.0.3 area 0

Router1(config-router)# network 192.168.10.132 0.0.0.3 area 0

! Router2 OSPF

Router2(config)# router ospf 1

Router2(config-router)# network 192.168.10.64 0.0.0.31 area 0

Router2(config-router)# network 192.168.10.124 0.0.0.3 area 0

Router2(config-router)# network 192.168.10.128 0.0.0.3 area 0

! Router3 OSPF

Router3(config)# router ospf 1

Router3(config-router)# network 192.168.10.96 0.0.0.15 area 0

Router3(config-router)# network 192.168.10.112 0.0.0.7 area 0

Router3(config-router)# network 192.168.10.120 0.0.0.3 area 0

Router3(config-router)# network 192.168.10.128 0.0.0.3 area 0

Router3(config-router)# network 192.168.10.132 0.0.0.3 area 0

6.  Connectivity Testing

Ping tests were performed in Cisco Packet Tracer between devices across all departments to verify end-to-end reachability after routing configuration.

| Source | Source IP | Destination | Dest IP | Result | Packets |
| --- | --- | --- | --- | --- | --- |
| PC0 (Sales) | 192.168.10.1 | PC2 (HR) | 192.168.10.65 | Success ✓ | 5 / 5 |
| PC0 (Sales) | 192.168.10.1 | PC4 (IT) | 192.168.10.97 | Success ✓ | 5 / 5 |
| PC0 (Sales) | 192.168.10.1 | PC6 (Finance) | 192.168.10.113 | Success ✓ | 5 / 5 |
| PC0 (Sales) | 192.168.10.1 | PC8 (Mgmt) | 192.168.10.121 | Success ✓ | 5 / 5 |
| PC2 (HR) | 192.168.10.65 | PC4 (IT) | 192.168.10.97 | Success ✓ | 5 / 5 |
| PC4 (IT) | 192.168.10.97 | PC6 (Finance) | 192.168.10.113 | Success ✓ | 5 / 5 |
| PC6 (Finance) | 192.168.10.113 | PC8 (Mgmt) | 192.168.10.121 | Success ✓ | 5 / 5 |
| PC8 (Mgmt) | 192.168.10.121 | PC1 (Sales) | 192.168.10.2 | Success ✓ | 5 / 5 |

7.  Conclusion

This assignment successfully demonstrates the implementation of Variable Length Subnet Masking for a five-department organization network. Key outcomes:

- VLSM was applied correctly in descending host-count order, allocating subnets of /26, /27, /28, /29, /30, and three /30 WAN links.

- Efficient IP utilization — only 136 out of 256 addresses consumed, leaving 120 addresses free for future expansion.

- Five department LANs were created with right-sized subnets, eliminating the IP waste associated with Fixed-Length Subnet Masking.

- Management Department received a /30 subnet (same size as WAN links), which is the tightest possible allocation for 2 hosts.

- Two manual static routes were configured per assignment requirements for Sales ↔ HR connectivity via the R1–R2 WAN link.

- OSPF Area 0 was deployed on all three routers for dynamic route propagation across the remaining subnets.

- All devices successfully pinged across departmental boundaries, confirming full end-to-end network reachability.

The Cisco Packet Tracer .pkt file containing the complete configured topology is submitted alongside this report.